# Electronic Questions:

Pumping Test Analysis Module
Developed by Thomas Reimann (TU Dresden), Eileen Poeter (Colorado School of Mines), and the iNUX Team (2025).

# Preface

This assessment introduces the fundamental concepts underlying pumping test analysis in groundwater systems. It focuses on the physical principles of groundwater flow toward a pumping well and covers theory, analytical solutions, and practical interpretation. Accordingly, the questions are designed to assess and strengthen understanding of the general characteristics of transient flow to wells, the Theis solution for confined aquifers, the Hantush-Jacob solution for leaky aquifers, and the Neuman solution for unconfined aquifers. Emphasis is placed on interpreting groundwater responses to pumping and recognizing how aquifer properties and boundary conditions influence observed behavior.

The assessment can be used:

- as an initial diagnostic tool before working with pumping test models,

- as a learning aid while studying analytical solutions and their assumptions, or

- as a consolidation exercise after hands-on modeling or data analysis activities.

The assessment is implemented in the Pumping Test Analysis Module. It serves as a self-assessment before working through the module, while working through the module's exercises and applications, and when finishing a section. On the subsequent pages, the questions are provided. A version with feedback (depending if the answer was correct or not) and solutions is available on request.

# Questions: Theory Section

1. What is the value of hydraulic head within a confined aquifer?

A. Lower than the elevation of the aquifer top
B. Equal to the elevation of the aquifer top
C. Higher than the elevation of the aquifer top
D. A confined aquifer doesn’t show a hydraulic head

2. What is considered as transient?

A. A system with a constant long-term average
B. A system in which head changes over time.
C. A system in which head is different at different locations
D. A system experiencing water abstraction.

3. What parameter is used to describe the ability of an aquifer to transmit water?

A. Storativity
B. Hydraulic head
C. Transmissivity
D. Specific storage

4. What is the equivalent to 0.001 m³/s?

A. 0.1 Liter per second
B. 1 Liter per second
C. 10 Liters per second
D. 100 Liters per second
E. 1000 Liters per second

# Questions: Transient Flow toward a well in a confined aquifer

5. What conditions are applicable for use of the Theis equaiton?

A. Steady state flow, confined aquifer
B. Transient flow, confined aquifer
C. Steady state flow, unconfined aquifer
D. Transient flow, unconfined aquifer

6. If there is no recharge to an aquifer how does the cone of depression behave with ongoing time?

A. The cone of depression will reach a steady state
B. The cone of depression continue to increase
C. The cone of depression is not dependent on time
D. The cone of depression will decrease

7. How does storativity (S) influence the response of an aquifer to pumping?

A. A higher storativity results in a slower drawdown response
B. A higher storativity leads to more rapid flow to the well
C. Storativity only affects steady-state conditions
D. Storativity is not relevant for confined aquifers

8. Which of the following statements describes the drawdown at a point due to pumping in a confined aquifer?

A. It decreases with time
B. It increases with time
C. It remains constant over time
D. It is independent of the pumping rate

9. How does the drawdown change at one specific distance from the well if storativity is decreased?

A. Drawdown is less
B. Drawdown is more
C. Drawdown is not affected

10. If the estimated transmissivity T is too low, how will the predicted drawdown compare to the true drawdown?

A. The predicted drawdown will be too small
B. The predicted drawdown will remain unchanged
C. The predicted drawdown will be too large
D. The predicted drawdown will be too large close to the well and too small far from the well

# Questions: Theis’s parameter estimation

11. What conditions are appropriate for use of the Theis Solution?

A. Steady state flow, confined aquifer.
B. Transient flow, confined aquifer
C. Steady state flow, semiconfined aquifer
D. Transient flow, semiconfined aquifer
E. Steady state flow, unconfined aquifer
F. Transient flow, unconfined aquifer

12. How does storativity S influence the response of an aquifer to pumping?

A. A higher storativity results in a slower drawdown response
B. A higher storativity leads to more rapid flow to the well
C. Storativity only affects steady-state conditions
D. Storativity is not relevant for confined aquifers

13. How does the drawdown change at one specific place and time if the transmissivity is increased?

A. The drawdown is less
B. The drawdown is more
C. The drawdown is not affected
D. All of the above depending on the parameter values

14. Which of the following assumptions was made in the development of the Theis solution for transient flow to a well?

A. The aquifer has variable thickness
B. The aquifer is confined and infinite in lateral extent
C. The well fully penetrates an unconfined aquifer
D. The pumping rate varies with time

# Questions: Hantush-Jacob parameter estimation

15. What conditions are appropriate for use of the Hantush-Jacob Solution?

A. Steady state flow, confined aquifer.
B. Transient flow, confined aquifer
C. Steady state flow, semiconfined aquifer
D. Transient flow, semiconfined aquifer
E. Steady state flow, unconfined aquifer
F. Transient flow, unconfined aquifer

16. What is the key difference between the Hantush-Jacob solution and the Theis solution?

A. Theis solution applies to leaky aquifers, while Hantush-Jacob is for unconfined aquifers
B. Hantush-Jacob solution accounts for leakage through an aquitard, while Theis assumes a fully confined aquifer
C. Theis solution considers the presence of an impermeable boundary
D. Hantush-Jacob solution applies only to steady-state conditions

17. What parameter is introduced in the Hantush-Jacob solution to account for leakage?

A. Storage coefficient
B. Leakage factor (B)
C. Specific yield
D. Transmissivity

18. Which of the following best defines the leakage factor B in the Hantush-Jacob solution?

A. A measure of the storage capacity of the aquifer
B. A parameter that describes the rate at which water moves through the aquifer
C. A term that quantifies how much water leaks through an aquitard into the aquifer
D. A constant value independent of aquifer properties

19. What happens to drawdown in a leaky confined aquifer compared to a fully confined aquifer?

A. Drawdown decreases more rapidly in a leaky aquifer
B. Drawdown is less in a leaky aquifer due to the additional water source
C. Drawdown remains the same in both cases
D. Drawdown is greater in a leaky aquifer due to the additional water source
E. Drawdown occurs only in the aquitard, not in the aquifer

20. How does the Hantush-Jacob solution represent storage in the aquitard?

A. Aquitard storage is ignored
B. Aquitard storage controls the rate of leakage
C. Aquitard storage controls the timing of leakage

# Questions: Neuman parameter estimation

21. What conditions are appropriate for use of the Neuman Solution?

A. Steady state flow, confined aquifer.
B. Transient flow, confined aquifer
C. Steady state flow, semiconfined aquifer
D. Transient flow, semiconfined aquifer
E. Steady state flow, unconfined aquifer
F. Transient flow, unconfined aquifer

22. In an unconfined aquifer, what additional factor must be considered when analyzing well drawdown?

A. The presence of a confining layer
B. The effect of specific storage
C. The delayed response due to water table storage
D. The transmissivity remains constant

23. What is the key difference between the Neuman solution and the Theis solution?

A. Theis solution applies to unconfined aquifers, while Neuman is for confined aquifers
B. Neuman solution accounts for delayed water table response, while Theis assumes an immediate response
C. Theis solution considers leaky conditions, while Neuman assumes a fully confined system
D. The Neuman solution is only valid for steady-state conditions

24. What additional parameter is introduced in the Neuman solution to account for water table effects?

A. Leakage factor
B. Specific yield (Sy)
C. Hydraulic head
D. Transmissivity (T)

25. How does the specific yield (Sy) affect the response of an unconfined aquifer in the Neuman solution?

A. A higher specific yield results in a slower drawdown response
B. A lower specific yield leads to more rapid flow to the well
C. Specific yield does not influence drawdown in unconfined aquifers
D. A higher specific yield causes the drawdown to increase over time

26. Compared to a confined aquifer, what is a key effect of pumping in an unconfined aquifer?

A. The drawdown is larger due to the lower storativity
B. The drawdown is delayed due to water table storage effects
C. The drawdown is smaller because of the lower transmissivity
D. The aquifer does not experience any delayed response

# Questions: Exercise and Application of Pumping Test Analysis

27. What conditions are appropriate for use of the Theis Solution?

A. Steady state flow, confined aquifer.
B. Transient flow, confined aquifer
C. Steady state flow, semiconfined aquifer
D. Transient flow, semiconfined aquifer
E. Steady state flow, unconfined aquifer
F. Transient flow, unconfined aquifer

28. What is the main goal of fitting pumping test data to curves describing drawdown in response to pumping?

A. To determine unknown aquifer properties from observed data
B. To predict the future pumping rate of a well
C. To measure the depth of the groundwater table
D. To calculate the hydraulic gradient

29. If the estimated transmissivity (T) is too low, how will the predicted drawdown compare to the true drawdown?

A. The predicted drawdown will be too small
B. The predicted drawdown will remain unchanged
C. The predicted drawdown will be too large
D. The predicted drawdown will be too large or too small depending on the distance from the well

30. What role does curve fitting to pumping test data play in the process of estimating aquifer parameters?

A. It predicts future rainfall effects on groundwater levels
B. It determines the total groundwater storage in the aquifer
C. It estimates the pumping rate required for full aquifer depletion
D. It helps visually compare theoretical and observed drawdown curves

31. Why is it important to compare predicted drawdown using estimated parameters with the true drawdown?

A. To determine the maximum sustainable pumping rate
B. To evaluate the accuracy of the parameter estimation process
C. To find out if the aquifer is unconfined or confined
D. To measure the recharge rate of the aquifer

32. Which parameters are estimated using the Theis solution and pumping test data?

A. Hydraulic Conductivity and Porosity
B. Transmissivity and Storativity
C. Specific Yield and Permeability
D. Pumping Rate and Aquifer Thickness